SOURCE* Handbook

# A large-scale benchmark tool

# for road networks

# in developing countries

# User’s guidebook

# June 2001

# * Standard Overall Ultralite Road Care Estimate

- The SOURCE project was initiated and conducted

- by the RMI (Road Management Initiative),

- a component of the SSATP (Sub-Saharan Africa Transport Policy Program),

- which is a joint program administered by the World Bank, in conjunction with road sector donors and RMI partner countries.

- As part of this project, SSATP has requested ISTED’s assistance

- to develop a series of tools:

- this handbook, the general booklet and the SOURCE CD-ROM.

- The SOURCE CD-ROM, which contains all the field tools

- (including the handbook) together with the dedicated softwares,

- is available free of charge on request.

- Contacts

- SSATP, RMI:

- Internet address: http ://www.worldbank.org/afr/ssatp

- E-mail: ssatp@worldbank.org

- Fax: (1) 202 473 80 38

- Address: SSATP, The World Bank, 1818 H Street NW,

- Washington, DC 20433 USA

Sub-Saharan Africa Transport Policy Program (SSATP)

The World Bank and Economic Commission for Africa

- ISTED – Road Department:

- Internet address: http ://www.isted.com

- E-mail: isted@i-carre.net

- Fax: (33) 1 40 81 23 31

- Address: ISTED, La Grande Arche, Paroi Sud,

- 92055 Paris La Défense Cedex, France

This handbook is the baseline methodological guide

for applying the SOURCE method.

Before beginning the measurement campaign,

it must be fully assimilated by each crew leader.

Chapter 1 concerns the preparation of the campaign

and is essential both for the coordinator and the crew leaders.

Chapter 2, which is particularly intended for the crews,

describes the field measuring procedure.

Chapter 3, on data processing, primarily concerns the coordinator.

The handbook must be on-board the vehicle throughout

the measurement campaign,

to check on a point in the method,

resolve a hard case,

or illustrate an explanation given to the driver

by the crew leader.

| CONTENTS OF THE HANDBOOK AREA OF VALIDITY............................................................................................................ BASIC PRINCIPLES...................................................................................................... CHAPTER 1: PREPARATION OF THE MEASUREMENT CAMPAIGN - DEFINITIONS................................................................................................................. #### 2. DRAWING UP THE MEASUREMENT CAMPAIGN PLAN..........…............................... - General.................................................................................................................... - 2.1. Items provided by the client.............................................................................. - 2.2. Division into sections....................................................................................... ## 2.3. Optimizing the campaign plan under constraints.............................................. - 2.4. Finalizing the campaign plan............................................................................ - A SOURCE tool: the network map editor................................................................. 3. CREW TRAINING AND PRACTICE................................................................................ - 3.1. SOURCE accreditation..................................................................................... - 3.2. Crew training.................................................................................................... - 3.3. The clearance to start the field campaign........................................................ - 4. EXTERNAL QUALITY CONTROL.................................................................................. 4.1. The crew's instructions..................................................................................... 4.2. The two external control procedures................................................................ - 5. CALIBRATION PLAN...................................................................................................... 5.1. Calibration program............................................................................................ 5.2. Setting up the permanent calibrator.................................................................. - 5.3. Ordinary calibration procedure............................................................................ 5.4. Precautions due to accidents affecting calibration............................................. 5.5. Post-connecting precautionary calibrators.......................................................... 5.6. Using a topometer: guidelines............................................................................ #### 6. CREWS AND LOGISTICS.............................................................................................. ## General................................................................................................................... ## 6.1. Crew member profiles....................................................................................... - 6.2. Vehicle.............................................................................................................. - 6.3. Instruments....................................................................................................... - 6.4. Documentation................................................................................................. - 6.5. Stationery......................................................................................................... - 6.6. Administrative aspects..................................................................................... | - p. 8 - p. 9 - p. 14 - p. 16 - p. 16 - p. 16 - p. 18 - p. 20 - p. 22 - p. 23 - p. 24 - p. 24 - p. 24 - p. 25 - p. 26 - p. 26 - p. 26 - p. 27 - p. 27 - p. 28 - p. 29 - p. 31 - p. 32 - p. 33 - p. 35 - p. 35 - p. 35 - p. 36 - p. 36 - p. 37 - p. 38 - p. 39 |
| --- | --- |

| - CHAPTER 2: SPEED/TRAFFIC MEASUREMENT PROTOCOL - DEFINITIONS................................................................................................................ - 2. GENERAL SPEED/TRAFFIC MEASURING PRINCIPLE................................................ General................................................................................................................... 2.1. Behaviour of the floating vehicle....................................................................... 2.2. The operator's work.......................................................................................... #### 2.3. General conditions of speed/traffic measurements.......................................... - 3. CREW AND ROAD USERS............................................................................................ General.................................................................................................................... 3.1. Parking............................................................................................................. 3.2. Driver's rest periods.......................................................................................... 3.3. Reactions of followed motorists........................................................................ 3.4. Refuelling......................................................................................................... ## 4. BASIC PROTOCOL ....................................................................................................... A SOURCE tool: the input form set......................................................................... 4.1. Using the forms................................................................................................ 4.2. Identifying section ends.................................................................................... 4.3. Wait at the start of a section............................................................................. 4.4. Pursuit of the first LV........................................................................................ 4.5. Hare-tortoise alternation rule............................................................................ 4.6. Stopping at the end of a section....................................................................... 4.7. The 90 km/h rule............................................................................................... 4.8. The 15 minute rule............................................................................................ 4.9. The 15/10/5 minute rule.................................................................................... ##### 4.10. The abandoned LV rule ................................................................................... 4.11. Re-starting in LV wait position........................................................................ 4.12. Taking break periods into account.................................................................. - 5. DIFFICULTIES IN FOLLOWING A LV............................................................................ ##### 5.1. The followed LV parks or turns off......................................................………… 5.2. Multiple-stop LV categories.............................................................................. 5.3. Abnormally slow LVs........................................................................................ 5.4. Dangerously fast LVs....................................................................................... #### 5.5. Other difficulties in following an LV..................................................................... 5.6. Lack of any LVs: FV travelling solo.................................................................. | - p. 41 - p. 46 - p. 46 - p. 46 - p. 47 - p. 47 - p. 48 - p. 48 - p. 48 - p. 48 - p. 48 - p. 49 - p. 50 - p. 50 - p. 50 - p. 51 - p. 52 - p. 55 - p. 55 - p. 57 - p. 59 - p. 62 - p. 64 - p. 64 - p. 65 - p. 66 - p. 68 - p. 68 - p. 68 - p. 69 - p. 69 - p. 70 - p. 71 |
| --- | --- |

| ##### 6. ROAD AND TRIP RELATED DIFFICULTIES................................................................. General.................................................................................................................... 6.1. Village crossings............................................................................................... 6.2. Diversions......................................................................................................... 6.3. Multi-lane roads................................................................................................ - 7. TRAFFIC DIFFICULTIES................................................................................................ General.................................................................................................................... 7.1. Convoys............................................................................................................ 7.2. Congestion and traffic jams.............................................................................. 7.3. Other traffic difficulties...................................................................................... 8. WEATHER DIFFICULTIES............................................................................................. General.................................................................................................................... 8.1. Precipitations.................................................................................................... 8.2. Passability drops.............................................................................................. - 9. HOW TO HANDLE DIFFICULTIES: RECAP................................................................... Solution Tree (diagram and comments) ................................................................. - 10. PREPROCESSING OF MEASUREMENTS.................................................................. General.................................................................................................................... 10.1. Why preprocess? .......................................................................................... 10.2. General presentation of preprocessing........................................................... 10.3. The two preprocessing stages: sorting and calculations................................ 10.4. First sort: grouping the sequences into packets............................................. 10.5. Second sort: determining invalid packets....................................................... 10.6. Third sort: identifying non re-measurable packets.......................................... 10.7. Second stage: calculations and conclusions.................................................. 10.8. Preprocessing: a self-inspection and control tool........................................... - 11. RE-MEASURING........................................................................................................... 11.1. U-turns during measuring runs....................................................................... 11.2. Re-measuring after the end of the measuring run.................…………………. | - p. 74 - p. 74 - p. 74 - p. 76 - p. 79 - p. 81 - p. 81 - p. 82 - p. 84 - p. 85 - p. 87 - p. 87 - p. 87 - p. 88 - p. 89 - p. 91 - p. 92 - p. 92 - p. 92 - p. 92 - p. 92 - p. 93 - p. 94 - p. 96 - p. 100 - p. 102 - p. 103 - p. 103 - p. 103 |
| --- | --- |

| - CHAPTER 3: DATA PROCESSING - 1. PRIOR CONTROL AND VALIDATION............................................................................ - General................................................................................................................... 1.1. Controlling before transferring.......................................................................... 1.2. Checking for errors........................................................................................... 1.3. Validating preprocessing choices .................................................................... 1.4. Who controls? ................................................................................................. - 2. DEFINITIONS................................................................................................................. - 3. COMPUTERIZED PROCESSING................................................................................... 3.1. Moving on to the computer .............................................................................. A SOURCE tool: the calculation workbook............................................................. 3.2. Using ordinary workbook tools......................................................................... - 4. TRANSFERRING CALIBRATION DATA........................................................................ 4.1. Transferring data from the calibration sheets................................................... 4.2. Detection of errors........................................................................................... 4.3. Step-by-step guide .......................................................................................... - 5. TRANSFERRING LINK DATA........................................................................................ 5.1. Transfer order................................................................................................... 5.2. Transfer method............................................................................................... 5.3. Detection of errors ........................................................................................... 5.4. An option: runs "to be ignored"......................................................................... - 6. INTERMEDIATE CALCULATIONS AND REVIEW.......................................................... 6.1. Simultaneous calculations................................................................................ 6.2. Network recap sheet......................................................................................... 6.3. Checking aid: the mono-link review sheet........................................................ - 6.4. Rectifying traffic levels...................................................................................... - 7. OUTPUTS....................................................................... ................................................. 7.1. Country summary sheet..................................................................................... 7.2. Quality sheet....................................................................................................... 7.3. Speed-traffic maps...........................................................................…............... | - p. 106 - p. 106 - p. 106 - p. 106 - p. 106 - p. 107 - p. 108 - p. 115 - p. 115 - p. 115 - p. 116 - p. 117 - p. 117 - p. 117 - p. 118 - p. 119 - p. 119 - p. 120 - p. 121 - p. 122 - p. 123 - p. 123 - p. 123 - p. 125 - p. 127 - p. 129 - p. 129 - p. 129 - p. 130 |
| --- | --- |

AREA OF VALIDITY

The following methods have been developed for trunk road networks of developing countries, particularly African countries. They are appropriate for networks with the following characteristics:

Network structure

- Few motorways, few sections with more than 2 lanes

- Mixed paved / unpaved roads

- Deteriorated general condition

Range of speeds

- Normal traffic speeds much slower than standard speeds in the West. Mostly below 90 km/h.

Range of traffic

- Traffic levels usually low to very low compared with standard levels in the West. Mostly fewer than 700 light vehicles per way per day. But at least a few major links exceeding 350 LVs per way per day.

- Networks generally far from saturated (except peri-urban links).

The SOURCE method makes a series of statistical approximations carefully adjusted and validated for such contexts, on the basis of major experiments on a very varied sample of African networks.

But it would be unwise and sometimes quite wrong to use them in completely different contexts.

BASIC PRINCIPLES

The method consists in making standardized measurements of overall travel speeds and traffic levels of a well-defined category of vehicles, i.e. "light vehicles", over a reference network that is well-identified throughout the country. These field data are then aggregated to produce a standardized game with country-wide macro indicators that give an overall picture of road network performance levels and suitably combine: integration of the actual service level (through traffic speeds) and integration of the service provided, i.e. generation of road transport (through traffic volumes).

#### An ultralight method for accurate results

To develop this game with national macro indicators, a meta-databank has been built to cover all major road links, for which the measurement campaign is able to provide comprehensive input. Pre-existing road databanks and traffic surveys are used solely to better organize the measurement campaign but their data are not fed into the processing chain. In fact, the method disregards pre-existing resources so that the results are certain to be standardized and relevant to countries less well covered by ordinary road management instruments.

This gives an instant photograph of exceptional statistical quality, of the status of a country’s main road network, by means of an ultralight measurement campaign.

The definition of this photograph (the mesh pitch of this meta-databank) is perfectly tailored to methods for determining road policy and for road management planning and performance monitoring. Precise programming of work and daily network operations require finer meshing and other assessment methods.

This method gathers an abundance of results, while minimizing the costs of the measurement campaign and the logistical constraints.

General presentation of the floating operation method

The measuring method used – the "floating" method – consists in integrating an ordinary light vehicle into the traffic stream (the measuring vehicle, i.e. the floating vehicle). This vehicle will alternately follow a fast light vehicle (by which it has been overtaken) and a slow light vehicle (with which it has caught up) subject to a few adjustments. The actual speed of the floating vehicle over the travel path is then used as a result, after various corrections.

A single run, in a single way of travel, will usually be enough.

Simultaneously, the floating vehicle keeps a count of traffic travelling in the opposite way (“apparent” traffic). With various corrections, this gives both a series of speeds and a series of traffic counts. Both aspects of this basic information on network performance are thus combined.

Theoretically the very precise measuring protocol assigned to the floating vehicle makes it a “statistical integrator” that gives high-quality results. This is the key to the method. It means that once is enough for this “living” statistical integrator (so to speak) to travel the entire network under review, at speeds close to common speeds, without fixed facilities or instrumentation systems.

#### Precautions in order to obtain representative measurements

The speed to be measured is the “free” speed of light vehicles i.e. over a significant length – their spontaneous speed when not impeded by any particular disturbance. At the same time, the traffic level encountered must remain representative. This requires certain precautions such as the following:

- All measurements must be made on business days and in business hours.

- Any interfering factors liable to adversely affect traffic and impede speeds or flows during measuring operations are to be avoided or circumvented according to strict rules.

Lasting reasons for reducing speeds, due to the condition of the road itself (any defects in an ageing, inadequately maintained roadway) are not in themselves considered as disturbances. Through their influence on speeds, this poor condition of the road is precisely what we are aiming to determine.

The measurement campaign is always conducted in the dry season (because we want to rate the network during the period of optimum practicability).

The reference network

It is the client of the measurement campaign that determines the network to be covered, based on its own problems within the framework of the national road management policy.

In the case of Sub-Saharan Africa, to ensure that comparisons are relevant over time on the one hand, and from one country to another on the other hand, a sound, simple, objective method has been developed to determine country-wide reference networks. These SOURCE reference networks form essential samples that in fact usually correspond to the hard core of priority networks defined by the incumbent authorities. This methodology for defining SOURCE reference networks, together with a permanent Observatory System for the whole of Sub-Saharan Africa, is not described in this handbook which is devoted to the measuring method1.

Frequency of measurement campaigns

The ordinary periodicity recommended for this type of country-wide network monitoring (i.e. the periodicity of SOURCE measurement campaigns) is once every three years. This frequency meets recommendations made by worldwide specialized bodies and implemented in developed countries, and it is naturally homogeneous with the conventional life cycles of major road networks.

More intense monitoring may be required in certain cases. Special requirements, such as the objective progress indicators for the implementation of a national road maintenance and management policy, may cause a rate of one campaign per year to be adopted. The client will then be a major network manager-operator, or the responsible political authority, or again a multipartite body of the “Road Maintenance Fund” type, in charge of mobilizing funds for maintenance and accounting to road users for their satisfactory application. The SOURCE method, which provides systematic monitoring of changing patterns of travel times on major routes, is used as a basic tool for dialogue between road users and road managers.

Still more in-depth monitoring of changes on a single major road link (or a specific sub-network such as the overall portion treated under a rehabilitation program), is also possible using the SOURCE method, but it requires some precautions where distances are too short. Below a threshold of 150 km, for instance, the required statistical quality will only be achieved by increasing the number of measuring runs.

For daily network monitoring by the front-line manager, it is possible to derive more summary methods from the SOURCE method, which yield direct in-situ estimations of common speeds and common traffic levels over a given route, made “on the move”, without any special preparation or training. In this case, the aim will be to determine rough orders of magnitude, for guidance only, which will characterize the pattern of change of a given road link (whatever the season). This ultra-simplified method, which provides valuable information at virtually zero cost but with no real statistical quality, is not covered by this handbook2.

The SOURCE handbook and tools

This handbook therefore only covers the actual SOURCE method corresponding to country-wide measurement campaigns which ensure that SOURCE quality standards are met.

To make it easy to understand, we have taken the most common case. We have opted for a client (the authority that places the order for the measurement campaign), a consultant (the service provider required to implement the campaign) and a controller (another service provider to whom the client assigns the task of controlling the quality of this campaign). In fact, these three assignments may sometimes be performed by the same network management agency (in which case it is essential to distinguish between them and separate them properly). In this case, the reader will make the necessary adjustment.

Just as an opinion poll has to comply with extremely precise sampling rules if it is to yield significant results, SOURCE measurements, based on a statistical method, must comply with a controlled, standardized margin of error that provides reliable, reproducible results.

The specifications to be applied in the field are consequently severe, even though statistical integration at the network level will tolerate a lower quality of measurements on lower-trafficked road links.

SOURCE work protocols are demanding for field operators. Their work is made easier by appropriate tools (detailed data sheets, etc), which are also designed to facilitate rigorous quality control at two levels: “internal” by the service provider in charge of the campaign, who is responsible for field measurements and data processing; “external”, by the client or a third party mandated for this purpose. This quality control protocol requires prior certification of each measurement crew.

The SOURCE handbook provides all these specifications and is accompanied by the relevant tools. It details strict rules and instructions for preparing the measurement campaigns, making the measurements and processing the data.

CHAPTER 1

PREPARATION OF THE MEASUREMENT CAMPAIGN

1 - DEFINITIONS

THE CHAIN OF PARTNERS

Consultant : Service provider made responsible for the measurement campaign by the client, pursuant to a contract.

Client : Body that orders the measurement campaign from the consultant. It may be a local authority, a professional organization, an independent organization, etc.

Controller : Representative of the client or any person acting on behalf of the client to supervise and/or provide external quality control of the measurement campaign conducted by the consultant.

Coordinator : Representative of the consultant, acting as the client’s partner, in charge of managing the campaign: preparation, supervision and coordination of the measurement crews, data processing.

Operator (or crew leader) : Engineer or senior technician in charge of a measurement crew consisting of himself, the driver and the measuring vehicle.

THE NETWORK

Reference network : The "reference network" is the network of roads or portions of roads to be subjected to the campaign of speed and traffic level measurements entrusted to the consultant.

Link : Major constituent element of the reference network. Each link usually corresponds to a portion of road in the country's trunk network.

The links to be covered are determined in advance by the client.

Link rank : The links of a reference network can be broken down into classes or ranks depending on their economic or strategic importance. The classification (from rank 1 to rank 4) depends on objective criteria not specific to a country but specific to the SOURCE method (and designed for African countries).

The series of link ranks to be covered is communicated by the client.

Section : To organize the measurement campaign properly, each link of the network must be divided into sections. Each section must be more or less homogeneous as regards its physical characteristics and traffic levels. This means that a section cannot include two road portions of different rank. The recommended length of a section is between 30 and 100 km.

Speed and traffic measurements are made on a section-by-section basis. The results are then grouped together per link during data processing.

The sub-division of the network into sections (based on the prior division into links) or its revision (based on the division into sections from a previous campaign) is the consultant’s responsibility.

## Sub-section : On the road, measuring is sometimes disturbed due to unexpected reasons or circumstances. When making measurements, the relevant section must therefore be redivided into several sub-sections, one of which corresponds to the extent of this disturbance.

A sub-section thus results from the redivision of a section, made necessary for any unexpected reason when making the actual measurement.

EQUIPMENT

Cyclometer (or odometer) : a mechanical device to measure short distances on foot, which is simple, inexpensive and takes up little room in a vehicle boot. It looks like a bicycle wheel fitted with a handle and a mechanical recorder. It is used on road worksites and by surveyors.

Topometer : a small electronic device to measure longer distances on-board a vehicle. It includes a sensor that is fixed in the middle of the cable of the standard recorder, and a calculation and display device on the dashboard. The sensor measures the number of turns of the recorder cable, usually by magnetic impulses. The topometer thus provides the same measurement as the standard kilometric recorder but with greater reading accuracy. It also enables the correction coefficients (calibration) to be recorded. The displayed measurements are already corrected. A topometer fitted on a vehicle is usually left in place permanently. To recover it, it is necessary to replace the recorder cable, which has had to be cut to mount the topometer.

## Floating vehicle (FV) : a perfectly ordinary vehicle used by the crew in charge of measuring by the “floating “ method.

#### 2 – DRAWING UP THE MEASUREMENT CAMPAIGN PLAN

The client usually communicates to the consultant the reference network divided into links, in a computerized form (Excel workbook) that enables maps and charts to be edited.

A skeleton map is then drawn up by the consultant, which also shows the predetermined sections with their approximate lengths and their code numbers.

2.1 – Items provided by the client

Case of a network with classification of links into ranks

When the network to be addressed is the SOURCE reference network as defined for the requirements of the Observatory, the links are classified on the basis of “ranks”. The rank rating of each link is determined according to objective, unchanging criteria common to all countries in Africa.

Throughout Africa, the link ranks range from 1 to 4, in decreasing order of their role in economic trading. For a given country, no more than three ranks are ever adopted for the SOURCE reference network: ranks 1-2-3 for countries with at least one link in rank 1 (ranks of major links), ranks 2-3-4 for smaller countries where no rank 1 road exists. Some small countries do not have any rank 2 etc. either. The ranks will never be lower than rank 4.

The method of determining SOURCE reference networks is described in a separate document3.

Note: For a given country, this SOURCE reference network may change marginally, usually in three-year periods: possible inclusion of new routes, upgrading or downgrading of some links. There is consequently a prior exercise to update or check the reference network, usually already performed earlier by the client. When there has been a change in the reference network, the comparison of SOURCE results from one period to another must naturally integrate some elementary precautions.

It will often be advisable to use a single SOURCE measurement campaign to meet both priority network monitoring requirements corresponding to the national policy, and requirements resulting from an Africa-wide comparative observation based on the SOURCE reference network. The network to be covered, possibly under an agreement between several partners, will then be extended beyond the SOURCE reference network itself.

Whatever the case, it is assumed here that the client has previously determined the network to be covered, hereafter called the “reference network”, and that it will not have to be changed by the consultant.

Whether the link in a network is rank 1,2, 3 or 4, or an additional link outside the SOURCE reference network (and therefore without a specific rank), this will have no practical effect on the working method of the measurement campaign. The measuring method is the sole method and it is not affected by the rank.

Neither is there any problem when the network defined and communicated by the client has a different type of classification (or no classification at all).

General Rule: List of items given to the consultant by the client

- identification elements of the reference network to be covered: map of the network, identification of links by origin-destination, classification of links by rank (if any, just for the record), administrative codes of roads and tracks corresponding to these links with their approximate distances.

- the indication of specific sub-sets to be complied with for exploding the final processing results of the campaign (in the reference network, e.g. the actual SOURCE reference network; the priority network defined at national level; the sub-network coming under a specific local or regional authority; etc.).

- any essential items from a previous SOURCE campaign for all or part of the network: measurement campaign plan, division into sub-sections, identification of beginning- and end-of-section landmarks, and as appropriate, section data sheets, data processing files, etc.

List of items to be procured by the consultant

Whether he receives these items from the client or obtains them from the incumbent authorities, it is also essential for the consultant to procure the following:

- Full, detailed identification of the portions of this network subjected to major roadworks during the past 3 years, namely: (1) major roadwork programs: new building, reconstruction, strengthening and rehabilitation  (2) periodical maintenance programs.

- The corresponding ongoing roadwork programs

- Programs for roadworks planned within the year, where available.

This is because each of these three work program categories may entail specific decisions on section divisions for the planned measurement campaign.

It is usually the consultant’s task to gather additional information such as any available traffic statistics and any records, even partial, of the road condition.

SOURCE tools

- This practical floating operation handbook, with its annexes: specimens of section data sheets and pages, kilometric recorder calibration sheets, etc.

- Drafting and display software for the reference networks (map editor), enabling automatic output (including a complete library of SOURCE reference networks in the 48 countries of Sub-Saharan Africa).

- The SOURCE data processing workbook. After the results are processed, they can be restored to the initial software for the production of standard maps illustrating network-wide speeds and traffic, in hard copy or electronic form.

The SOURCE tools are all available in English and French versions on the SOURCE CD-ROM.

They are free of rights of use, subject to the credit line and compliance with the specified limits of theoretical use. These reserves are detailed in the CD-ROM, the terms of which are authoritative.

As the map editor and data processing workbooks are simply Excel files, the consultant only requires ordinary hardware such as a standard PC with Windows (95 or more recent) and Excel (95 or more recent).

2.2 – Division into sections

Reference Network

The reference network road links must be divided into homogeneous sections (as regards both traffic and main road characteristics).

It is the consultant’s task to make this division into sections based mainly on cartographic information, his experience and his knowledge of the network.

Exception: If any such divisions have already been made for a previous measurement campaign of the same type, the client will have informed the consultant of the fact. No change must then be made, unless otherwise specified by the client.

However, in some cases, the consultant may propose that some previously used sections are more finely divided up (e.g. to take account of recent or planned asphalting of part of an old section).

In principle, measuring on a section will be made in the same way as for the previous campaign. Any change on this point must be specially recorded in the measurement campaign plan submitted for the client’s approval.

Once the division into homogeneous sections has been prepared and finalized, it is to be submitted to the client before the actual start of the campaign. Any changes made necessary during the campaign will be duly specified afterwards.

The reference is the skeleton map of links (preferably prepared using the map editor), which the consultant will have finalized by adding the division into sections. This map must accompany each operator. He will complete it as necessary and include any division adjustments that may be required in the course of the campaign.

What is a section?

The length of a section must not exceed 100 km. But a section must not be too short either (it is advisable for the minimum distance to be 30 km).

However, even before dividing up the section, a link in the reference network may itself be shorter than 30 km, owing to the geography. In this case, this link will naturally form a single section. This exceptionally short section must be taken into account with its actual length and included in the measurement campaign.

When must a section be changed?

Each of the following criteria generally justify the creation of a new section limit:

- lasting change of a paved road section into an unpaved road section;

- lasting change in the number of lanes or other very clear change in road width (e.g. a road usually 7 m wide is widened to 5.5 m over 30 km);

- clear, lasting change in the type of alignment, usually corresponding to a geographic break that greatly modifies the landscape crossed (e.g. change from a relatively flat, straight road in the plains to a winding valley road or a succession of steep slopes;

- heavy foreseeable variations in traffic (e.g. major junction, particularly where one of the sections, but not the other, is subject to congested traffic at certain times;

- clear, lasting change in road conditions (e.g. recent improvement on the section, with major roadworks);

- crossing through a big city (with major traffic interchanges).

Theoretically, no section must be less than 30 km long. This rule for the minimum length will prevail. It may cause one of the 6 above criteria for a change of section to be disregarded.

Exception: Solely for the paved / unpaved criterion, shorter sections, down to a minimum length of 10 km, will be acceptable. Mixed paved / unpaved sections disturb the calculation of most final SOURCE results. Such mixed sections are consequently to be particularly avoided.

Special case: hazardous areas, restricted traffic areas

Where links are subject to severe traffic restrictions, whether official or de facto, owing to serious banditry, disturbances for different reasons, war or civil war, the consultant must identify them in his network division and suggest that they are excluded from the measurement campaign. Even when there is no clear risk to the safety of the measuring crews, this will nonetheless be the technical attitude that should be proposed whenever traffic is non-existent or is subject to special organization: severe time restrictions, traffic organized in convoys, whether or not they are escorted. This is the case for spontaneous precautionary measures or official measures (of course, such convoys do not travel at a statistically operable free speed).

The SOURCE method is by nature inapplicable to non-trafficked or restricted-traffic routes, for whatever reason.

Other special case: major long-term road closures

One of the major links in a reference network determined by the client may be subject to a major closure, with no possibility of a local diversion (because of a problem or an accumulation of problems, such the sweeping away of a succession of bridges). Either the through traffic is diverted onto a radically different alternative route (that may be outside the reference network) or the region is cut off (for lack of any generally passable alternative route).

Sometimes such major closures will have existed for years.

What is to be measured? The two half-roads on either side of the closure point, solely reduced to local traffic? The dominant alternative route (where it exists)? This must be discussed on a case-by-case basis with the client when preparing the campaign.

## 2.3 – Optimizing the campaign plan under constraints

When drawing up the measurement campaign plan, several types of constraints must be integrated:

SOURCE constraints

- The measurement campaigns must be organized during the dry season.

- In the event of long rainy periods that hold up operations or if a long portion of the network is made practically impassable owing to the weather, the campaign will be postponed, in whole or in part, to a later date (subject to the client’s agreement).

- The speed measurements must be made section by section in a single way of travel. Theoretically this way does not matter and is up to the consultant to decide (but in principle it will be in the same way as for the previous campaign, where relevant). For each section, the reference measurement way (once determined) is called the “outward way” and the corresponding measuring run is called the “outward run”.

- If the network geometry requires return trips or connecting trips between two links undergoing measurements, these trips must be made while remaining on the reference network, preferably using heavily-trafficked links.

All these return trips and connecting trips must give rise to a second measurement operation, whenever the traffic recorded on the previous trip has exceeded 100 LVs per hour and per way of travel. The corresponding measuring run is called the "return run" (in the “return way”).

- Measurements are only made during the daytime. Measuring will begin one hour after sunrise and will end no later than one hour before sunset (hours during which road users' behaviour is not modified by darkness).

- The time on the road to be scheduled for the measurement phases must take into account the lower productivity in the event of light traffic.

Caution: The lower the traffic level, the longer it will probably take to make the measurements, because of the FV wait times required by the method. The productivity of the method falls with the traffic, from one time the technical travel speed for well-trafficked links to more than twice this speed for very low-trafficked links. This must be carefully taken into account.

Field constraints

- Make provision for places to stop at mid-day, overnight or on rest days;

- Take into account public holidays and similar holidays;

Measurements must only be made on business days (not holidays) so that the traffic will be normal. It will not therefore be possible to make measurements on Sundays in most countries and it may or may not be possible on Fridays and Saturdays depending on local rules and customs fixing non-business days4 (and labour regulations applicable to the crew).

- Take into account all local events, whether exceptional or recurring, liable to disturb or greatly modify traffic in the period or on the day of measurement on each of the links;

For example: regional fairs and markets; large-scale pilgrimages or religious holidays; evacuation of harvested produce; etc.). It is up to the consultant to make the most of the available information in this respect and take the necessary precautions.

- Allow travelling time for simple connecting trips where applicable;

- Take into account the kilometric recorder calibration program, which requires a passing through the capital every fortnight, for an ordinary calibration test using the permanent calibrator (see same chapter, parag. 5.1 “Calibration program”).

Flexibility constraints

- Allow a sufficient flexibility margin for travelling times to take into account, in addition to the technical measurement time, interruptions of all kinds required by the method, technical breaks, various different incidents (particularly weather-related) and any U-turns for immediate restarting of suspect measurements, etc.

- In addition, allow half-days in reserve at the end road branches of the network or at certain strategic intersections, to enable measuring of certain sections or sub-sections to be repeated as cost-effectively as possible, without upsetting the measurement campaign plan, where measurements are poor for any reason, or to make up for lost time (due to heavy rain, for example).

2.4 – Finalizing the campaign plan

Depending on the network to be covered, the available time and the client's order, and to take into account all the foregoing requirements and constraints in drawing up the campaign plan, it may be necessary to form several crews who will share the measuring work.

The presence of several crews will naturally be used to optimize the campaign. In particular, a judicious choice of the circuits of each crew can limit the number of connecting trips.

For each crew, a very precise campaign plan is drawn up and given to the client for validation. This plan includes:

- all information on the crew (identity and task of crew members, description and registration of vehicle, etc.);

- the skeleton map of the network, indicating the links to be measured, with their approximate lengths and codes, etc.;

- a work schedule in the form of a summary chart indicating, ½ day by ½ day, the sections to be measured, (names, codes, lengths, ways of measurement), places for overnight stopovers and rest days, flexibility margins, etc.

- if the network has undergone previous measurement campaigns, add the series of descriptions of beginning- and end- of-section landmarks from the previous campaign: place names, full description, exact kilometerage between landmarks (this information is to be found in the series of section data sheets. See Chapter 2, parag. 4.2. “Identifying section ends”).

Once validated by the client, this plan must be followed scrupulously by the crew. Any deviation must be subject to further validation by the client according to a precise procedure (a priori or a posteriori procedure depending on the circumstances). See Chapter 1, parag.4.1.

A SOURCE tool: the network map editor Characteristics This is an Excel file of around 14 MB before compression, contained in the SOURCE CD-ROM, that can be copied onto your hard disk. It is named "SOURCEMapEdit.xls" (a single bilingual version). The software components are activated as soon as the file is opened (it is simply an Excel file, format 97, with standard macrocommands in Visual Basic for Excel). You can call up a "command panel" which will guide you securely right through to all the functionalities. Tools This editor enables you to: - create and edit graphically your choice of road network links for your SOURCE measurement campaign. - store the results of the SOURCE measurement campaign (imported from the dedicated calculation workbook in a light interchange format. See Chapter 3 "Data processing"). - generate and consult the corresponding interactive result maps (jclick and interrogate), - print them or export inert images as illustrations. To begin to generate network graphics, you simply import an ordinary road map as an image file from a scanner (in any usual format). You then use the mouse to re-draw your network as an overlay on this imported background map. A quick, simple process. This editor integrates a library containing all the SOURCE reference networks for countries of Sub-Saharan Africa and offers personal libraries to manage your own networks or customized versions of the networks provided. Specific features - Capacity: solely limited by the operating capacities of your computer (available memory size and system resources). The number and complexity of the country bases to be stored (number of links, cities, boundary and coast portions) are not limited. It is advisable that your computer has a good clock rate – e.g. 466 MHz – and at least 64 MB RAM. - This editor is fully bilingual (it changes from English to French and vice versa at your request, both for screen dialogue with the user and for editing maps and other by-products). - The editor's main help button calls up a main built-in interactive help sheet. - A whole series of special help elements is also accessible by icons displayed at various stages on the active components of the “command panel”. This map editor is provided as a facility but it is not essential to the use of the SOURCE method.

## 3 – CREW TRAINING AND PRACTICE

3.1 – SOURCE accreditation

Each crew leader (operator) must be "SOURCE-accredited", to prove he has fully mastered the method, both the theoretical aspects (assimilation of the handbook) and the practical aspects (field use).

SOURCE accreditation is awarded by one of the controller’s representatives when the operator passes specific tests. This preliminary accreditation does not exempt the operator from the training day.

3.2 – Crew training

A prior training phase, with field work on a sample of roads (at least one day) must therefore be scheduled before starting the measurement campaign.

The sample of roads is not necessarily taken from the reference network. In any case, the measurements made during training must not be taken into account when processing the data of the actual campaign.

The operator and the driver

It is essential for the driver and the operator chosen for the actual crew to practice the method together. Much of the training consists in bringing their practical work methods into line with each other (dictating kilometric recorder data, identifying vehicles to be followed, giving instructions to stop quickly, etc.) and getting the procedures to work smoothly so that the techniques are perfectly ready for the first kilometre of actual measurements.

Insofar as the driver is a salaried employee of a vehicle rental agency, not directly of the consultant in charge of the measurement campaign, it is important to ensure that he will be available throughout the campaign and for the training day, and to check his real capacities besides driving, and his motivation. He is not being asked to perform an ordinary job. The quality of the measurements will depend greatly on the way he works.

Several crews

When several crews are involved in the same campaign, it is important to arrange meetings between them to enable them to compare and harmonize their use of the method. It is advisable for this practice day to be held jointly so that all the crews (2 or 3) can meet in situ and review their practice work together at the end of the day.

This will be particularly beneficial for discussing procedures and any resourceful solutions developed, or to ensure that instructions have been interpreted properly and homogeneously, in order to avoid risks of non-quality.

Briefing / Debriefing

Whether one or more crews participate in this training day, it is the coordinator himself who will hold a short briefing session in the morning and a more in-depth debriefing session at the end of the afternoon. Just before this group debriefing session, the operators will each have performed preprocessing of their data pack, which will then be reviewed and checked during the session by the coordinator. He will use this as a basis for any useful advice, clarifications and comments.

3.3 – The clearance to start the field campaign

The results of the training day (calibration sheets, data packs and interchange file of data transferred to the calculation workbook) will be communicated to the client (or to the controller acting on his behalf) before the measurement campaign actually begins. Together with each crew's measurement campaign plan, they will form the basic elements on which the client will draw to give the consultant clearance to start the measurements. This clearance is construed as a formal agreement for the entire dividing up of the network into sections and for the detailed schedule of the measurement campaign, which will then become contractual obligations for the consultant.

4 – EXTERNAL QUALITY CONTROL

During the measurement campaign, the crews undergo quality control. The application of this control requires the crews to comply closely with the following instructions.

4.1 – The crew’s instructions

- Compliance with the measurement campaign plan (both routes and timetables) defined half-day by half-day and duly approved by the client and/or the controller;

- Obligation of prior agreement from the client and/or the controller before making any voluntary change in the campaign plan

- Obligation of a immediate regularization with the client and/or the controller to record accidental deviations from the campaign plan. Immediate a posteriori information is mandatory.

Any change in the campaign plan without notifying the client and/or the controller would make it impossible for the controller to perform his task.

Whether or not there is intent to defraud and for whatever reason except force majeure, such change would thus be professional misconduct on the part of the operator and accordingly a breach of contract between the client and the consultant.

4.2 – The two external control procedures

Scrupulous compliance with the approved measurement campaign plan is mandatory and conditions normal implementation of the two external control methods given hereafter:

- Without notice, a controller placed in charge of quality control by the client may join the crew for a day or a half-day as a passive observer. In this case, he boards the measuring vehicle and, at his request, the crew will submit the data sheets and calibration sheets in its possession, together with any documents recording progress of the measurement campaign.

- Without notice, a controller placed in charge of quality control by the client may make kilometric recorder measurements on the same day and over the same route, in the form of a fully-equipped independent counter-measurement crew. In this case, should he so request, the crew submits the data sheets and the calibration sheets in its possession, together with any documents recording progress of the measurement campaign (at the end of the day or at any time during the day, as requested).

These two situations may occur at any time during the measurement campaign. They must not cause the crew to change the current day’s program or its working method during this observation phase.

5 – CALIBRATION PLAN

A topometer is not essential to measure distances with the specified accuracy: just by using your car’s standard kilometric recorder, satisfactory results can be achieved, providing a correction coefficient is subsequently applied to the unprocessed measurements, which is carefully determined by a calibration test as described below.

5.1 – Calibration program

Calibration tests on the kilometric recorder yield a correction to be applied to the unprocessed recorder measurements when the data are processed.

These test series are to be performed as follows:

- Ordinarily, at least once a fortnight, for which the “permanent calibrator” must be used (see parag. 5.2). It is mandatory for the measurement campaign plan to provide for at least one run every fortnight in the capital where this calibrator is situated.

- Exceptionally, after any work on the front axle of the FV: wheel or tyre change, reinflation, or any mechanical operation such as wheel rebalancing or alignment, etc.; and naturally after any vehicle change made necessary by mechanical or other failure.

As a rule, all these test series, whether ordinary or exceptional, will be performed at the location of the “permanent calibrator”.

Ordinary calibration tests: Theoretically, the coefficient obtained from the first series of ordinary tests is only applicable the first week, whereas the coefficients obtained from the following ordinary series of tests (the first after the first fortnight) will be applied to measurements of the current fortnight (in principle, the previous week and the following week). Recorder drift is assumed to be regular, barring any incident.

In fact, as ordinary test series are permissible on any day of each fortnight, this ideal configuration will accept any adjustments that prove necessary (fix deadlines for the validity of the coefficients halfway between the dates of the successive tests. Later, to process the data of a given measuring run, it is the starting time of the run that will determine the coefficient automatically applied by the calculation workbook.

Exceptional calibration test: This test must occur at the earliest after the incident making it necessary (work on axle or vehicle change). If it is postponed until a later date and a puncture occurs before the test, all chance will be lost of determining the right coefficient applicable to measurements made in the meantime, and they will have to be repeated.

The dates and times of these incidents (work on the front axle or vehicle change) will be carefully noted, as they will indicate the point after which the correction coefficient must be changed at the data processing stage.

If measurements are made while the FV is running on a spare wheel, the coefficient must be tested with the wheel before remounting the rim fitted with the repaired tyre or the new tyre.

Note: Why calibrate the kilometric recorder?

- Kilometric recorder measurements are never accurate, because they are based on the number of turns of the vehicle's wheels (counted by a mechanical or electronic method). They therefore vary with the types and pressures of the tyres. Experience has shown that the margin of this systematic error is around one percent in relative terms.

- However, kilometric recorders are predictable, i.e. for a given tyre type and pressure, the relative error is constant.

- The recorder display is inaccurate, as it only shows hundreds of metres. If this display is mechanical (the figures being shown on thumbwheels that continuously rotate between full integers), the reading is to the nearest twenty metres. This margin of error for the reading is constant in absolute terms, around 10 m.

Thus over a distance of 1 km, the measurement error and the reading error are in the same order of magnitude. However, over distances greater than 10 km, the reading step is of minor significance (it is still 10 m), whereas the inaccuracy of the measurement becomes more substantial in absolute terms (more than 100 m). But that inaccuracy is easily corrected.

It is thus easy to achieve the required accuracy, providing the correction coefficient, carefully determined by the calibration test, is applied to the unprocessed measurements..

5.2 – Setting up the permanent calibrator

Definition of the permanent calibrator

A single reference calibrator must be available, carefully determined and measured with an accuracy of 1 for 2,000. All the series of calibration tests, whether ordinary or exceptional, must be performed solely at this place, using this “permanent calibrator” (except those that must be performed in the field, by improvising a precautionary calibrator, to be subsequently post-connected to the permanent calibrator . See parag. 5.5. hereafter).

This calibration distance must consist of a paved, fairly straight road portion in good condition, at least 2,000 metres long, to be selected by the consultant close to the capital, using existing markers.

It will be advisable to paint the distance between the two markers on the back of each of them, with the agreement of the department in charge (e.g. "Calibrator: 2023 m >>" and "Calibrator << 2023 m". This will avoid having to look up this information in the documentation.

Unless there is an incident, it will be enough for the measuring vehicles to re-calibrate their kilometric recorders, solely at this location, once a fortnight. These are ordinary calibration tests.

Services of a surveyor for each national measurement campaign

The services of a surveyor for half a day are compulsory for determining the calibration distance, i.e. the exact distance between the markers of the permanent calibrator5. The surveyor will work according to accepted practice by any method he may choose to ensure accuracy of less than one metre, and he will make a report on this operation.

If there has been a previous national measurement campaign (in principle, three years earlier), where possible use the same permanent calibrator, but a surveyor is nonetheless required to restore the permanent calibrator. This is because it is impossible to be absolutely sure that the calibration distance markers will be relocated, or that they have not been moved in the meantime because of roadworks.

Whatever the country, to establish the permanent calibrator or any other calibrator, do not rely on:

- kilometric or decakilometric distance markers or any other road markers (you can never be sure of the exact distance between them even when they have been installed recently)

- kilometric distance indications on road maps (rarely up-to-date and often too inaccurate).)

All these procedures would introduce uncertainty inconsistent with the accuracy required of SOURCE campaigns.

5.3 – Ordinary calibration procedure

- The test procedure will be the same whether operating on the permanent calibrator or the precautionary calibrator (see next parag.). The test requires a calibration sheet to be created according to the model provided with this handbook. See Chapter 2, parag. 4.

The calibration test requires five runs, performed at any speed, at a time when traffic is low, to enable safe manoeuvering and unhampered driving without having to pull out.

- The correction coefficient to be adopted will be the mean value of the "exact distance / vehicle recorder distance" ratios of these comparisons. The model of the calibration sheet will guide the calculation. Unless an incident makes it necessary to re-calibrate, this correction coefficient is applied to all measurements of that current fortnight (therefore in part retroactively. See above, parag. 5.1).

This coefficient will be applied later (by the calculation workbook) to the unprocessed measurements transferred from the section data sheets, to derive the exact distance after data transferring data. The crew does not have to make these calculations in situ.

Calibration sheet

5.4 – Precautions due to incidents affecting calibration

In the event of an incident or work that is liable to modify the systematic error of the kilometric recorder (puncture, reinflation, etc.), the FV will immediately perform precautionary calibration in situ., on a well-chosen, well-identified portion of road.

The aim is to determine a temporary reference distance, the “precautionary calibrator”. The method is the same as for determining the permanent calibrator, but it is inevitably  improvised. It is chosen on a fairly straight, paved portion of road in good condition. The calibrator must be between two pre-existing distance markers (or other permanent landmarks). The distance between them must be certain to exceed 2,000 m.

The calibration test is then performed as indicated in parag. 5.3 above, using the same model of the calibration sheet. What differs is that the exact distance between the markers is not known at the time of the test.

Practical case: how to note an incident on the data page and data sheet

(to be studied during the second reading of the handbook)

On the current data page, the incident must be recorded very clearly. A “Break” sequence will inevitably correspond to this incident. This sequence will end this page and will mark the point of coefficient change for subsequent data processing. It will be necessary to use a new data page  for the next measurements.

Furthermore, exceptionally, even if the next measurements on the section are made without delay, the operator, when pre-processing data at the end of the day, will make out two distinct data sheets for this sole data pack, each one referring back to a different calibration sheet number:

- the data sheet covering the measuring period up to the incident, to which the previously valid correction coefficient will be applied later. It refers back to the ordinary calibration sheet that was valid before the incident,

- the data sheet covering the measuring period after the incident, to which a new correction coefficient will be applied later. It refers back to the precautionary calibration sheet.

The ad-hoc box on the model of the section data sheet will be marked with a cross to show that the same run is exceptionally recorded on two complementary data sheets6.

If no further incident occurs before next passing through the capital:

This precautionary calibration will simply be disregarded and the kilometric recorder will be recalibrated subsequently over the calibration distance. The correction coefficient then obtained will be applied as from this incident.

If a second incident occurs in the meantime:

The precautionary calibration must then be used. This precautionary calibrator must be post-connected to the permanent calibrator, as follows.

5.5 – Post-connecting precautionary calibrators

Providing the two end landmarks of the precautionary calibrator are carefully indicated on the calibration sheet (for subsequent re-identification), measurement of the real distance between these landmarks may be postponed if necessary.

Two situations may occur:

- Immediate post-connecting: The crew of the floating vehicle is equipped with a cyclometer and must immediately measure the real distance between these two improvised landmarks. However, this measurement will only be used in case of further incident.that makes it necessary.

- Deferred post-connecting: The crew is not equipped with a cyclometer (frequent case). If necessary, the exact distance between the two landmarks can be measured later using one of the following three methods:

- bringing a cyclometer to the spot

- with the same or a different vehicle, using its kilometric recorder appropriately calibrated on the permanent calibrator

- with another vehicle equipped with a topometer appropriately calibrated on the permanent calibrator

As stated in parag. 5.4 above, it rarely proves necessary to perform this deferred post-connecting.

Post-connection techniques

- For immediate post-connecting, the exact distance between two landmarks, which is measured using a cyclometer, requires at least three measurements to be made. Each time the cyclometer has been used for this purpose, it must subsequently be tested against the permanent calibrator (also 3 measurements), and where necessary, the corresponding measurements will be corrected. Post-connecting is effectively performed against the permanent calibrator, by means of the cyclometer.

The cyclometer must also be calibrated or checked frequently.

- When deferred post-connecting is performed using the kilometric recorder of another vehicle, the mean value of at least ten runs must be determined. Calibration of its recorder will have been performed no more than fifteen days earlier (this is mandatory after any work on this vehicle’s front axle), based on the permanent calibrator, also in ten runs. This is because any errors may build up.

- When deferred post-connecting is performed using another vehicle fitted with a topometer, the mean value of at least five runs must be determined. Its topometer will have been calibrated no more than 15 days earlier (this is mandatory after any work on this vehicle’s front axle), by five runs based on the permanent calibrator as indicated in the following parag. 5.6.

5.6 – Using a topometer: guidelines

If a topometer is fitted on the vehicle, it will considerably improve reading accuracy compared with a kilometric recorder – (from an accuracy of around ten metres at best to a guaranteed accuracy of one metre).

Using a topometer does not make calibration tests unnecessary. They must be performed according to a program that follows the same rules and always using the permanent calibrator.

Practical calibration procedure

It will be sufficient to apply the instructions on the manufacturer’s data sheet provided with the instrument. This type of calibration consists in making successive runs during which the applicable correction coefficient in the calculator of the topometer itself is directly stored, so that subsequently displayed measurements are already corrected.

Therefore, if you have a correctly calibrated topometer (based on the permanent calibrator), the distances read will not require subsequent correction as the correction is already integrated into the values read from the instrument. No coefficient has to be noted down.

By definition, the integrated calibration of the topometer is considered valid for all measurements during the fortnight following the test (no retroactivity in this case). In fact, except for incidents affecting calibration, the only coefficient to be applied later by the workbook for data processing will be 1 throughout the campaign.

Frequency of calibration tests

The frequency of ordinary tests does not change because a topometer is used. It is still one series per fortnight, to be performed on the permanent calibrator.

Incident affecting calibration

After any incident affecting the vehicle’s front axle, which requires immediate recalibration of the topometer, you must also immediately perform a series of calibration tests on a precautionary calibrator, for which there is likewise no change. Proceed as follows.

Practical method of precautionary calibration

Above all, do not change the internal calibration settings stored in the topometer (otherwise your instrument will display pseudo-metres that are probably even less accurate than those it displayed previously, which is not desirable). In fact you are not required to touch the topometer.

Simply measure your precautionary calibration distance with your topometer (calibrated as is). Until the next standard calibration to be performed on the permanent calibrator, you will now continue measuring by reading from the topometer and transcribing the inaccurate figures on your sheets, which will subsequently be corrected at the data processing stage.

Thus during this period, exceptionally, you will use your topometer as a kilometric recorder, with a different correction coefficient of 1, determined subsequently. It will be calculated on a calibration sheet, with a view to subsequent correction of the distances recorded on the data sheets (instead of these unprocessed distances being final, which is usually the case with a topometer). This will not detract from the accuracy, which will be much better in the long run.

Just as in the general case without a topometer, the precautionary calibrator can simply be disregarded if no further incident occurs before the next ordinary calibration. But if a further incident occurs in the meantime, the precaution will prove useful. Thus the precautionary calibrator will have to be post-connected, i.e. re-measured exactly once your topometer has duly been recalibrated.

#### 6 – CREWS AND LOGISTICS

Besides preparing the plans for measurements, calibration and training of the crews, the preparatory phase must be used to gather together all the equipment and documents required for the measurements.

Here is a list to be checked before the crew takes to the road so that nothing will be forgotten.

6.1 – Crew member profiles

The method requires the presence of two people on-board the measuring vehicle:

- 1 experienced driver (with experience on all types of roads) who will assist the operator by providing him with information during the trip (kilometric recordings, etc.) and who is able to assimilate and accurately apply the driving rules given by the handbook, according to the operator’s instructions.

- 1 measurement operator, the crew leader  (a senior technician or an engineer):

- with a good knowledge of the network,

- SOURCE-accredited and therefore able not only to correctly apply the ordinary rules laid down in this handbook but also to assess and analyse all types of situations requiring specific decisions, using the handbook or extrapolating its exceptional rules.

6.2 – Vehicle

Measurements will be made using a standard four-wheel-drive vehicle in good, safe running order, capable of travelling on all types of roads.

This vehicle does not require any special equipment (such as a radio transmitter).

6.3 – Instruments

No special instruments are necessary: no micro-computer or positioning system are required on board.

It is not essential to have a topometer as the accuracy of the kilometric recorder after calibration is sufficient.

Kilometric recorder

Distances are recorded by reading the vehicle's standard kilometric recorder. This recorder must therefore be fitted with a daily totalizing meter (with reset button) having a display in hundreds of metres (not only kilometres but also tenths of kilometres).

Be careful because a few standard pick-up models are not fitted with a totalising meter and are therefore not suitable.

With or without a topometer, the procedure will be practically the same. Only reading accuracy and comfort will change (the operator can read the topometer, whereas with the recorder, it is the driver who reads the kilometerage and dictates it to the operator to be recorded). For this reason, it is recommended (but not mandatory) to use a topometer.

Timer

Times will be recorded using a "sports" type digital timer with a memory.

Provide a spare timer in case of failure of timer (and batteries).

Clock or watch

Some time intervals should be measured using the vehicle’s clock or a watch.

6.4 – Documentation

Where possible, the following items must be available to the crew:

- The SOURCE handbook itself

- A small stock of kilometric recorder calibration sheets

- A sufficient stock of section data sheets with their data pages (provide plenty), together with file boxes to store them before and after use

- The crew’s campaign plan (see details in this chapter, parag. 2.4), comprising:

- the network skeleton map delivered to it, with the division into sections

- its detailed work schedule

- where necessary, the full series of descriptions of end-of-section landmarks as defined for the previous SOURCE campaign.

- a set of road maps to provide valuable information for using the method (travel distances, junction locations, built-up areas and road types)

- Other useful items of information on the network, where available, such as:

- the mean traffic levels recorded elsewhere on each section (if possible, to be converted into light vehicles or private vehicles per hour for a single way of travel, not taking trucks into account. This corresponds to the definition and unit used in the SOURCE method.)

- Information on truck traffic, which will enable some disturbances due to the traffic structure to be anticipated.

6.5 – Stationery

Clipboard

It is practical for the operator to have a clipboard to make recording easier on the data sheets.

Sundry items

A stapler will be necessary to assemble the data packs after they have been pre-processed (the section data sheet followed by the corresponding pages).

Provide all necessary items such as pens, etc. (nothing should be noted in pencil).

Pocket calculator

A pocket calculator (if possible able to do additions in hours and minutes) is essential at the stage of preprocessing the data, to be performed at the end of each measurement day.

6.6. – Administrative aspects

Certification

It is advisable for each crew to have a certification issued by the authority in charge of the road network at the national level (Public Works or equivalent Ministry). This certification will save time in the event of controls by the police or any other authority and can even reassure any road user who may be puzzled or worried by the behaviour of the floating vehicle.

Other items: A sticker can be placed on the windscreen of the floating vehicle, to be read from 2 metres away and marked in the following manner: “Measurements on road condition. National campaign, month and year”. The stamp of the competent road authority can be added for information.

The purpose of this sticker, about 8 cm in diameter, is mainly to give first-hand information to any pedestrians approaching the stationary FV but it is not necessarily intended to be read from afar by other vehicles in the traffic flow.

List and particulars of people or services to be contacted where necessary.

Particularly people directly involved in supervising the campaign (coordinator, client and/or controller), local services in charge of network management (such as subdivisions or local public works agencies) together with any services dealing with facilities, practical aspects, safety or comfort (hotels, family, etc.).

Cash

Fuel and official tolls can be paid by coupons where they exist.

It is regrettably advisable in some countries for the operator to have a supply of cash for unofficial tolls, to drive through roadblocks without excessive delays or complications.

1 This methodology is detailed in a technical note entitled “Reference Network Methodology” contained in the SOURCE CD-ROM.

2 The simplified SOURCE method is detailed in a special file entitled “On the move measurements by the simplified SOURCE method”, contained in the SOURCE CD-ROM.

3 This methodology is detailed in a technical note entitled “Reference Network Methodology” contained in the SOURCE CD-ROM.

4 Note: for predominantly Muslim African countries, as Friday morning is not usually a morning off, Friday is consequently included

5 As a curvilinear distance with respect to the road axis, like all the road distances given in this handbook.

6 Later, when processing the data, this original section must be split up into two different sections (one before and one after the place of the incident),to be entered successively.