CHAPTER 3

DATA PROCESSING

1. PRIOR CONTROL AND VALIDATION

The coordinator recovers all the hardcopy packs of each crew from the operators (calibration sheets + section data sheets, behind which the corresponding data pages are stapled).

It is advisable to submit several sets of packs together throughout the measurement campaign so that data processing can begin at the same time. This will also detect any systematic method errors made by the crew, and enable them to be corrected without waiting for the end of the campaign.

1.1. Controlling before transferring

The person in the consultant's office in charge of transferring the data must first perform an dual control operation, that proves essential, to check for errors and validate choices.

Physical data transfer into the workbook, for each data pack or sheet, will not begin until the data have been entirely controlled (the date can be stamped on the validated sheet, with the checker's signature). No further use will be made of the paper records for data processing. (They can be stored until they are finally handed over to the client or the controller).

1.2. Checking for errors

This person will check the data pages (data entry areas and preprocessing areas) and the data sheet for each individual data pack.

The purpose is to detect errors in the protocol, field data entry, data transfer, calculations, etc., but without making any interpretations.

- Transfer and calculation errors: he corrects them immediately.

- Protocol errors ("field behaviour errors"): He must identify all the unprocessable sequences, neutralize them and redo the calculations, then check that the 10% critical level is respected. If not, the data pack must be rejected and the measurements repeated (unless another run is scheduled or already available and considered correct after checking.

## 1.3. Validating preprocess choices

Each individual data pack must be validated for the preprocessing areas, Refs. 3, 4 and 5. Whenever the preprocessing process has required a decision, a suggestion will have been made by the operator in the field (preprocessing area on section data pages) and taken into account for his own data sheet calculations.

They will now be examined thoroughly and objectively before validating.

If the checker rejects the operator's suggestion: he himself will redo the calculations on the section. In this case, he will record the new calculations by hand, in red, on a photocopy of the original data pack (which he will merely cross out in red), then he will restaple the two data packs, which will be kept together.

Which choices must be validated?

If we go back to the three sort levels described in chapter 2:

First sort level: consisted in grouping the sequences into packets and isolating the measuring sequences of suspect sub-sections in suspect packets ("S"). This was a mechanical operation that did not involve any appraisals. An error in this first sort would be a transfer error (see above, parag. 1.2).

But appraisals were required in the 2 following sort levels:

- Second sort level: it was decided to validate ("OK") or to neutralize  ("OK") each suspect packet ("S"). This corresponds to ref. 3 in the preprocessing area.

- Third sort level: it was decided to classify each neutralized suspect packet ("OK") on the one hand, and each “skip” sequence (“F”), on the other hand as re-measurable ("R") or non re-measurable ("R"). This corresponds to refs. 4 and 5 in the preprocessing area.

- 1.4. Who controls?

Transcribing the data is a crucial operation for good integrated internal control. This is why “data transfer” is NEVER a typist’s task, but an essential step to be entrusted to an engineer conversant with the SOURCE method.

Data transfer, performed in the consultant’s office by the coordinator himself or a close senior colleague, will be all the easier where the operators in the field have taken care with the pre-processing. Moreover, it will be helpful to associate these operators in the final review of controversial cases of neutralization of suspect sub-sections.

- The calibration sheets are controlled by a similar method, but more simply as no specific choices are necessary. The only requirement is to check for errors.

- External control

- From now on the hardcopy packs will only be used for external control. This will cover the same control points, such as: Were we really right to validate a particular suspect measurement? Or to classify a particular neutralized or suspended measurement sub-section as non re-measurable?

- If these decisions have been made and validated improperly, they will have prevented re-measuring (wrongly in this case, as they will have made it possible to remain artificially below the 10% critical threshold). This will economize on resources but to the detriment of quality.

- As this part of the external control is performed later, once the measurement campaign is finished, it is bound to be penalizing because any belated re-measuring required on this account (error) would generate extra costs for the consultant. It is much better to detect this type of problem during the internal control, so that re-measuring can be organized at the end of the campaign when all the field facilities are still on hand.

2 – DEFINITIONS

GENERAL CONCEPTS

Traffic level, sometimes simply termed traffic or traffic intensity.

By convention, in the SOURCE documentation, except where expressly specified otherwise:

The term traffic concerns sections (that are by definition homogeneous) or links (not usually homogeneous). It is given in a single way of travel, solely for light vehicles per hour. Before rectifying traffic levels (see further ahead) it is given for a specific time slot and a specific day of measurement. After rectifying traffic levels, it is given as a mean standard value for business hours and days in the dry season (conventionally: 8.00 to 17.00 from Monday to Saturday, except for special public holidays).

The traffic levels generally referred to in this handbook are thus expressed as light vehicles per hour (LV/h) per way of travel.

Caution: In general, outside the SOURCE context, the term "traffic" can refer to substantially different concepts. Depending on the circumstances, traffic can be expressed:

- at a given place on the road, or for a portion deemed to have homogeneous traffic (where the traffic at each point is assumed to be the same), or for any portion (in which case it is the “mean” traffic for this portion, calculated by the mean value of the traffic on sections presumed to be homogeneous, this mean being weighted by the corresponding lengths).

- for a single way of travel, or for both ways of travel added together,

- for a single specific category of vehicles (e.g. light vehicles), or for all vehicles (either by simply adding the specific traffic levels in each category without weighting, or by assigning specific weightings to each category, which for instance, enables all the traffic to be expressed as “private vehicle units” -pvu- ),

- per hour or per day (this is just a calculation unit used to express the value. But it does not refer to the period covered by the measurement),

- as a mean value measured for a given time slot of a given day, or as a mean value reconstructed for the entire year.

Before comparing two traffic levels taken from different studies or reports, it is therefore essential to ensure that they equate on the following 5 points: At what place or on what portion of road? For how many ways of travel? For what vehicles and, where applicable, with what weighting scale between vehicle categories? Using what time unit? And over what period?

Traffic volume

For a given link or section of road and for a given period, the term “traffic volume” is used to mean the product of its mean traffic level by its length. Traffic volumes are expressed as vehicles [multiplied] by kilometres and [divided] by hour or by day (in abbreviated form: veh. x km / h or veh. x km / d, or for light vehicles alone: LV x km / h or LV x km / d).

In the vocabulary of transport economists, these traffic volumes correspond to the “transport production” concept.

By convention in SOURCE documentation, without exception, traffic volumes will always be given for both ways of travel and solely for light vehicles.

###### Apparent oncoming traffic

This is the oncoming traffic as observed by the moving measuring vehicle. It is expressed as the number of vehicles per hour in a time slot and on a given trip path and is only of relevance for a section with a homogeneous traffic level. (The real oncoming traffic is traffic counted by one or more stationary observers and is expressed as the number of vehicles per hour in a time slot at a given place, or as a mean value on the section in this time slot).

Once we know the travelling speed of the measuring vehicle, we can easily reconstruct the real traffic from the apparent traffic. In the simple case of a measuring vehicle travelling at the mean speed of light vehicles in its own way of travel, the real traffic of oncoming LVs, as a first approximation, is equal to half the apparent oncoming LV traffic.

### Rectified traffic levels

The traffic level, as measured, depends on the time slot and the day of the week of making the measurements. It varies depending on whether they are made in peak or off-peak hours, or on peak or off-peak days. It must be corrected so that it will only reflect a standard business hour of a standard business day.

By convention specific to the SOURCE method, the standard mean traffic on a section given as having homogeneous traffic, is the mean value of the traffic over all standard business hours (i.e. 08.00 to 17.00) of standard business days (i.e. Monday to Saturday, excluding official holidays, but including ordinary semi-business days), during the dry season.

Note: for predominantly Muslim African countries, as Friday morning is not usually a morning off, Friday is consequently considered a business day.

At the data processing stage, general country-specific variation laws for LV traffic, over hours of a day on the one hand, and over days of a week on the other hand, will automatically be derived from the entire corpus of measurement campaign data. This will enable the measured traffic levels to be rectified to convert them into standard traffic levels.

However, the method does not enable any annual variation law to be derived. SOURCE rectified traffic levels thus remain representative of the dry season (deemed to correspond to the optimum practicability of the network during the year).

Harmonic mean of speeds

Speeds never combine arithmetically but always by using harmonic means, whatever the case: whether to calculate the mean of speeds of a sample of vehicles on a single section (simple harmonic mean), or to calculate the resultant speed of a single vehicle on successive sections (harmonic mean weighted by their lengths).

How is a harmonic mean calculated?

By and large, for a set of n Xi values (i varying from 1 to n), where their arithmetic mean, Xarith is:

Xarith = (1/n) x Sum (Xi), i.e. the ordinary mean,

Their harmonic mean, Xharm, is defined as:

1 / Xharm = (1/n) x Sum (1 / Xi)

The harmonic mean is the reciprocal of the arithmetic means of the reciprocals.

This is a conventional concept. Note that the simple harmonic mean is a standard function available in Excel (but this standard function cannot calculate weighted harmonic means, which are also used to calculate SOURCE indicators. See below).

CAUTION: Speeds cannot be combined like ordinary physical values! This will be easily understood from simple numerical examples ##### Case of 2 vehicles on a single section Here the correct mean of the 2 speeds S1 and S2 is not their arithmetic mean (S1 + S2) / 2, but their harmonic mean 2 / [ (1/S1) + (1/S2) ]. The harmonic mean and the arithmetic mean only coincide if S1 = S2. Two vehicles make a trip on the same portion, length L=100 km. The travel time of the first vehicle is T1=80 mn (i.e. 80/60 h), the travel time of the second vehicle is T2 = 100 mn (i.e. 100/60 h). The two measurements of the travel speed on this portion are therefore: S1 = 100 km / (80/60) h = 75 km/h, and S2 = 100 km / (100/60) h = 60 km/h To obtain a good estimation S of the travel speed on this portion, the natural reflex would usually be to average the results, which would give: S = (S1 + S2) / 2 = (75 km/h + 60 km/h) / 2 = 67.5 km/h But this estimation would be wrong, for the following reasons: The real physical value is the travel time. The mean travel time T completed by both vehicles is: T = (T1 + T2) / 2 = (80 mn + 100 mn) / 2 = 90 mn = (90/60) h The corresponding mean speed is thus: S = L/T = 100 km / (90/60) h = 66.7 km/h This is fairly close to 67.5…but that is no excuse….!

It is easy to check that S, calculated by this method, really does give the harmonic mean of the 2 speeds, weighted by the lengths. We know that S1 = L/T1 and S2 = L/T2, therefore: T1 = L/S1 and T2 = L/S2. And we have simply calculated: S = L / T = L / [ (T1+T2) / 2 ] i.e. : S = 2 x L / [ (L/S1) + (L/S2) ] or again: S = 2 / [ (1/S1) + (1/S2) ] And this is the very definition of the harmonic mean. Moreover, we clearly obtain: 66.7 km/h = 2 / [ (1 / 75 km/h) + (1 / 60 km/h) ] ##### Case of a single vehicle on 2 successive sections Here the correct mean of the 2 speeds S1 and S2 is not their weighted arithmetic mean (L1xS1 + L2xS2) / (L1+L2), but their weighted harmonic mean (L1+L2) / [ (L1/S1) + (L2/S2) ]. The weighted harmonic mean and the weighted arithmetic mean only coincide if S1 = S2. A single vehicle has made a single trip consisting of two portions placed end-to-end. If we assume that you obtained a speed S1 = 60 km/h on the first portion of length L1= 40 km with a resulting travel time T1 of 40 mn (i.e. 40/60 h), this will accordingly give: S1 = L1 / T1 = 40 km / (40 / 60) h = 60 km/h Then you obtained a speed S2 = 36 km/h on the next portion of length L2 = 60 km, with a resulting travel time of 100 mn, i.e. 100/60 h, which will accordingly give: S2 = L2 / T2 = 60 km / (100 / 60) h = 36 km/h The first reflex, to obtain the travel time over the 2 portions combined, is to determine an arithmetic mean weighted by the distances, i.e.: S = (40x60 + 60x36) / (40+60) = 45.6 km/h. But this is wrong, for the following reason: The travel time over the 2 portions combined is naturally 40 mn + 100 mn = 140 mn, and the total length is 40 km + 60 km = 100 km. The real travel speed over the entire length is thus: S = 100 km / (140 / 60) h = 42.9 km/h. Not too far different from 45.6…but that is no excuse…..! It is easy to demonstrate that S is the harmonic mean of the 2 speeds S1 and S2, weighted by the lengths. We naturally obtain: S = (L1+L2) / (T1+T2) where S1=L1/T1 and S2=L2/T2 whereby: S = (L1+L2) / [ (L1/S1) + (L2/S2) ] whereby: (L1+L2) / S = (L1/S1) + (L2/S2) and so: (1/S) = [(L1/S1) + (L2/S2) ] / (L1+L2). Moreover we clearly obtain: 42.9 km/h = 100 km / [ (40 km / 60 km/h) + (60 km / 36 km/h) ]

All this seems obvious once it has been demonstrated. But when you are on the spot, if you are not careful, you will be tempted to mistakenly apply arithmetic means to the speeds instead of harmonic means. So take care… SOURCE methodology dispenses with speed calculations in the field and only requires you to handle distances, times and counts. The consultant will calculate the speeds automatically on the workbook. This calculation is error-proof.

SOURCE INDICATORS

SOURCE indicators are national aggregates calculated for the entire reference network of the relevant country. Calculation of SOURCE indicators must solely be made on the basis of measurement campaign data meeting the quality criteria of the SOURCE method.

The data processing sequence performed by the consultant is described in this chapter (from parag. 4.1 onwards). It enables macro indicators to be calculated and viewed for the entire reference network, using utilities for Excel.

SOURCE primary indicator : This is the "common speed" of LVs on the reference network. This indicator is calculated as follows: harmonic mean of LV speeds, measured section by measured section, weighted by LV traffic volumes in both ways of travel (dry season, business times and days). This is termed the “common” speed because it is the most probable speed of an LV travelling on the network, chosen at random.

This indicator is expressed in km/h.

Thus: CommonSpd = 2waytrafvol / Sum (2wayTrafVoli / Si) CommonSpd = Sum (2wayTrafVoli) / Sum (2wayTrafVoli / Si) CommonSpd = Sum (2 x Li x TrafLeveli) / Sum [ (2 x Li x TrafLeveli) / (Li/Ti) ] therefore CommonSpd = Sum (Li x TrafLeveli) / Sum (Ti x TrafLeveli) Li being the length of section i (i varying from 1 to n), Ti being the mean travel time on this section, Si being the mean speed on this section, TrafLeveli being the mean traffic level observed on this section (traffic being presumed homogeneous), and 2wayTrafVoli being the corresponding traffic volume.

SOURCE secondary indicator : This is the "travel speed” of LVs on the reference network. This indicator is calculated as follows: harmonic mean of LV speeds, measured section by measured section, simply weighted by the lengths of these sections (dry season, business times and days).

This speed is called the “travel speed” because it is the resultant speed of an LV that has travelled once over the entire network, exactly adopting the corresponding common speed observed on each section.

This indicator is expressed in km/h.

We know that: TravelSpd = Sum(Li) / Sum (Ti) i.e. : TravelSpd = Sum(Li) / Sum (Li / Si) which is effectively the definition of the harmonic mean of the speeds weighted by the lengths, as we saw earlier.

Note that for a single section (deemed to have homogeneous traffic), “common speed” and "travel speed” are the same, by definition. No confusion is therefore possible, which is why, solely for a section, the term “common speed”, or even simply "speed" will henceforth be used unreservedly.

SOURCE tertiary indicator = This is the ratio of the SOURCE secondary indicator to the SOURCE primary indicator (travel speed divided by common speed). It accounts for the homogeneity of the reference network situation with respect to the two-fold distribution of speeds and traffic levels irrespective of the general levels at which the speed and traffic are established.

Over the reference network, as national maintenance and rehabilitation logically gives priority to more highly-trafficked links, the best speeds usually correspond to the best traffic levels. The common speed is thus always higher in practice than the travel speed. For this reason, in practice, the tertiary indicator of a reference network is always between 0 and 1.

This ratio (for the reference network) thus relates to the "sound and active" network (the best part of the network which carries the main volume of traffic): When the ratio is close to 0, it is the best part of the network that concentrates the traffic. The closer the ratio is to 1, the more homogeneous the situation throughout the network. (Conversely, if the ratio were to exceed 1 and to diverge from it, the worst part of the network would concentrate the traffic).

We know that: Ratio = TravelSpd / CommonSpd Now TravelSpd = Sum(Li) / Sum (Ti) and CommonSpd = Sum (Li x TrafLeveli) / Sum (Ti x TrafLeveli) where Ratio = [ Sum (Ti x TrafLeveli) / Sum (Ti) ] / [ Sum (Li x TrafLeveli) / Sum(Li) ] As you see, this ratio is also equal to the ratio of the mean of the traffic levels weighted by the travel times, to the mean of the traffic levels weighted by the lengths.

Caution: this sophisticated indicator must be carefully interpreted:

- Theoretically, the tertiary indicator will be 1, for instance, if the speed of the LVs, measured section by section, is exactly the same throughout the reference network (whatever the traffic distribution). But it will also be 1 if the traffic level is exactly the same throughout the reference network (whatever the speed distribution section by section). These are naturally textbook examples.

- For some specific sub-networks (for instance, the unpaved network or the network of non-national roads), the common speed may be much lower than the travel speed. Then the indicator for the relevant sub-network, quite the reverse, is greater than 1. This reflects road policy problems on this sub-network considered alone – as previously stated, the part of the network carrying the most traffic is the part in the worst condition.

- Owing to the calculation method of this indicator, it is not associative (there is not the least reason for it to be so). In fact, it is perfectly possible to have a paved network with an index greater than 1 and an unpaved network also with an index greater than 1,  which together form a complete network with an index lower than 1. Moreover, this is a frequent case.

3 – COMPUTERIZED PROCESSING

3.1. Moving on to the computer

Once the control of the data packs and calibration sheets is finished, the data can be processed in the consultant's office on a standard micro-computer.

A SOURCE tool: The calculation workbook Characteristics This is an Excel file format 97 (around 4.1 MB in size before compression) contained in the SOURCE CD-ROM, that can be copied onto your hard disk (or onto a diskette after compression). It is named "SOURCECalculBook.xls" It contains software components that are activated as soon as it is opened (they are standard macrocommands in Visual Basic for Excel). These software components are just utilities for navigating within the workbook or for more convenient data import and export, etc., as the core workbook is mainly based on simple calculation formulae recorded in the ranges of the workbook sheets. This ensures that the workbook will remain operational in its basic functions even if it is opened under an incomplete version of Excel without the Visual Basic extensions. It comes with a general help utility enriched with specific assistance, all accessible from icons in the file calculation sheets. These help facilities make the calculation formulae clear by proposing simple numerical examples. Tools This workbook is designed to process field data from a complete SOURCE measurement campaign. Basic data from the hand-written calibration sheets and data sheets delivered by the measurement crews are transferred to this workbook. The computational architecture will then: - check data consistency through a series of tests - rectify the measured traffic levels, which will thus be converted into standard traffic levels - generate the "network recap sheet" which presents the unprocessed data and basic results, link by link. It then aggregates them for various basic divisions of the network: entire keyed-in network, SOURCE reference network, paved sub-network, unpaved sub-network, sub-sets of ranks 1, 2, 3 or 4, or any other choice of sub-set. These results naturally include the 3 SOURCE macro indicators as well as conventional values such as lengths, mean traffic levels and traffic volumes. - give various simple graphic representations of these results via the "country summary sheet" - provide a set of automatic controls that reflect the quality of the measurement campaign and the statistical relevance of the results. The computing process, including the coherence and quality tests, is automatic and instantaneous. Specific features - Capacity: 16 calibration sheets / a network of 100 links / 11 sections per link / 21 runs per link / a total of 247 runs for the network. - This file comes in 2 versions, English and French. - A data file import/export utility transmits or receives country data sets in a light format (around 120 KB), transferable by E-mail or diskette. - This utility can also be used for data interchange between the English version and the French version. It enables the entire set of standard tables and graphics to be "translated" effortlessly into either language where necessary. - The same interchange format enables the results of a measurement campaign to be imported into the map editor to generate mapping of results (providing you already have the network base in your libraries). - The help buttons call up a main built-in interactive help sheet. From this help sheet or from the calculation sheets you have access to various example sheets. This calculation workbook is provided as a facility but it is not essential to the SOURCE method. It is more convenient for processing the data and greatly helps to make the final results more reliable.

3.2. Using the ordinary workbook tools

In all the workbook sheets, you have general access to the standard Excel tools (print, copy and paste, delete, etc.). Don't hesitate to use them.

But you will find a few limitations as some of the ordinary tools are automatically disabled. For instance, you can only select areas on a white background, which are your only data entry areas, because all the others are protected against writing and even selecting. Another example, your drag-and-drop function is disabled as soon as the file is activated, and will not be restored until it is deactivated. This is to prevent you from accidentally dragging formats or addressing of formulae. The workbook is consequently secure.

A special button, "Insert a run" (see parag. 5.2 further ahead), which enables you to go back to already complete link input and insert an additional run by opening a further column, is also intended to make the workbook secure. This automated process prevents any irregularity due to invalid handbook processes.

##### Back-up

If the workbook version on your hard disk is corrupted through incorrect processing, do not keep trying to correct the formulae. It is better to first export the data if they are intact, then to reload the initial version of the workbook from the CD-ROM and lastly to re-import any data exported just previously. This will restore a correct version.

- 4 – TRANSFERRING CALIBRATION DATA

- To make the calculations, it is first necessary to enter into the workbook the calibration coefficients resulting from ad hoc tests, which will enable the measured lengths to be corrected. The calibration sheets filled out by the operators during the campaign are used at this stage.

- 4.1. Transferring data from the calibration sheets

The data from all the successive calibration operations will be grouped together in the first sheet of the workbook, the "MultiCalibSheet", crew by crew in chronological order.

To prevent any errors, (particularly as rules differ depending on whether or not a topometer has been used) the workbook calculates and automatically displays the exact period of validity of each of the correction coefficients (in principle, this will correspond exactly to the manual calculation results entered on the sheets by the operators). If the 15-day limit between 2 calibration operations is exceeded, the workbook will continuously display a discrepancy message.

To correct each length, the workbook will subsequently apply the right coefficient automatically, with reference to the crew number and the dates and times of the start of the  measuring run.

4.2. Detection of errors

Most transfer errors (typing errors, discrepancies, etc.) are immediately refused by the workbook, which then displays a warning signal and useful information for detecting and correcting the error. The signal will remain on the screen continuously if you disregard it.

The same is true for some inconsistencies between data entries (if the chronological order of the calibration sheets is not observed, etc.).

4.3. Step-by-step guide

1

2

3

4

- Enter the campaign headlines.

- First transfer general and permanent data for crew #1.

- Successively transfer data from each calibration sheet for crew #1.

- Then transfer the same way all data for crew #2, etc.

5 – TRANSFERRING LINK DATA

The data from all the measuring runs of the campaign will be grouped together in the second sheet of the workbook, the "MultiLinkSheet", link by link for all the crews taken together.

- 5.1. Transfer order

- It is advisable to observe the numerical order of the section codes (e.g. L1, L2… up to L80) as this will give presentable summary tables directly without having to reclassify the links (a complex operation that presents risks for the calculation formulae). But link data can be entered in any order as this will have no effect on the workbook calculations.

- Reminder: Data transfer does not consist in just taking the sheets as delivered by the crews and recopying the relevant boxes directly. It is essential to first check each pack thoroughly (prior control and validation).

5.2. Transfer method

1

2

3

4

5

- Enter a new link

- Enter the full section series of this link

- Enter now the data of all the runs on any section of this link

- Then go to the following link and process the same way

- Exceptionally, you can decide to « ignore » a previously-entered run

Prerequisite: Group all the packs corresponding to the measuring runs on the sections of a link. Check them thoroughly.

1/ Enter a new link

In the top part of the sheet, simply mark the general data for the link (rank, etc.) and the number of sections. And state in advance the total number of measuring runs available for all these sections.

2/ Enter the entire series of sections of this link

In the same part of the sheet, note the successive section limits, in order, from the beginning of the link to the end (simply note its division into sections).

Note: There is only one designation for the end-of-section limit and the following beginning-of-section limit, even though the corresponding landmarks may be physically different: e.g. entry to and exit from a built-up area. See Chapter 2, Parag.4.2 “Identifying section limits”. If this single designation is unsuitable, adopt a dual designation, with a hyphen.

3/ Now transfer the data for each of the runs on this link

Each column of the bottom part of the sheet corresponds to a measuring run, i.e. a section data sheet. Transfer the data on each run, including at the top the number of the section measured during this run.

The order in which the runs are entered is unimportant. You can even mix them or completely reverse the order: first run 1 on section 3, then run 2 on section 1, etc. The series of runs entered in the bottom part of the sheet does not have to conform, column by column, to the series of sections entered at the top (the division into sections). For each run, the fact of indicating the section’s number will be enough to generate a program link. The workbook itself will allocate the right weighting to each run, based on the number of available runs on a given section (e.g. 0.5 and 0.5 for an outward and return run).

4/ Then proceed in the same way with the next link

A special button always enables a column to be re-opened subsequently to enter an additional run into already complete link input.

5.3. Detection of errors

The workbook detects and immediately warns of various types of input errors, omissions or data inconsistencies. If, for instance, no measuring run has been made on a section division, or if the entered measurement date does not come within the period of calibration coverage for this crew, or if the ending time is before the starting time, or if the network measurement time is greater than the time slot [ending time – starting time], or if the length of a section measured on the outward run differs significantly from its length measured on the return run, or if the road has been recorded as paved on the outward run and unpaved on the return run, etc.

In some cases, the errors detected are indirect or too complex to be clearly described in the "MultiLinkSheet". In this case, a warning is nonetheless displayed, which suggests referring to the MonoLinkSheet" to clarify the problem (see further ahead).

5.4. An option: runs "to be ignored"

For each run, a special data entry area (under the designation "To be ignored?") enables this run to be totally eliminated (without deleting it) from all calculations made subsequently. If you mark a "X" in this area, it will effectively be ignored. If you remove the "X", it will be reintegrated.

If you mark "to be ignored" for the only run available on a section, this will naturally be refused by the workbook.

You may have to use this facility to optimize the final quality of the results. The aim in this case is to eliminate any lower-quality runs on a section with several runs. If you have an outward run with a "negligence" rate of 8%, and two return runs with a rate of 1% and 2%, you may decide to ignore the first lower-quality run (the "negligence" rate is the value to be compared to the critical threshold of 10%, i.e. the percentage of the re-measurable length not re-measured out of the total re-measurable length). It is under this name that it will appear in the workbook.

This possible elimination of some poor excess runs is NOT automatic as in fact the reasons for the poor quality must be closely examined and a decision taken. It therefore remains manual and reversible.

6 – INTERMEDIATE CALCULATIONS AND REVIEW

The calculations are made in the network recap sheet in real time as the data are progressively transferred. It is thus possible to review and control various intermediate and partial results before accessing the final results.

6.1. Simultaneous calculations

The automatic calculation made by the workbook is not sequential. It is made simultaneously (for each new entry in a data entry area). Thus each time the data entry for a new link is complete, right from the first link, the workbook immediately displays in the intermediate and final sheets, the calculations and results corresponding to all the previously entered links.

As long as you are dealing with data entry from the first links and all the already available traffic observations are insufficient for the traffic level rectification algorithm to give relevant results, a signal mentioning this problem is continuously displayed. It will automatically disappear when this coverage becomes complete (see parag.6.4 below “Rectifying traffic levels”).

6.2. Network recap sheet

"RecapSheet" displays the characteristic results and values for each of the links and systematically aggregates them for the basic network divisions: the network as a whole, its paved sub-set, its unpaved sub-set, and its rank sub-sets 1, 2, 3 or 4.

These results naturally integrate the 3 SOURCE macro indicators beside the conventional values: length, mean traffic level and traffic volume.

By default, calculations of indicators are made automatically on the SOURCE reference network (sorted into ranks, see chapter 1, 2.1), whereas all the available links are recapped column by column.

If you need to replace them by your own choice of links, whether to recap or to calculate indicators, a special data entry area enables you to select the links of your choice by entering their codes.

2

1

- Choose your own link series

- Calculation at network level (automatic)

A special button will also call up all the available links directly, both to recap and to calculate indicators (in this case performed without sorting into ranks).

The titles and designations of the indicator blocks always make error-proof reference to the set of links they cover, depending on the operative option.

These options are designed to enable you to monitor not only the SOURCE reference network but also your own national priority network or any other sub-set of interest to you (e.g. the network of roads administered by local or regional authorities, etc.).

6.3. Checking aid: the mono-link review sheet

"MonoLinkSheet" is a facility to isolate viewing of all the data and results from a given link. It is particularly useful for bringing to light a data entry problem or a significant inconsistency, as it displays some intermediate calculations for the sections, where the problem is sometimes more visible.

It contains a single data entry area in which to enter the code of the chosen link. The entire page is immediately brought up to date.

It is possible to "call up" all the links successively or not to use this facility at all.

1

2

3

- Choose your link and view it

- Calculations at run level (automatic)

- Calculations at link level (automatic)

6.4. Rectifying traffic levels

General presentation

The workbook directly makes a simple statistical analysis of all the valid traffic observations made during the campaign, from which it derives a dual rectification key: H-rectification (hour-based rectification) then D-rectification (day-based rectification).

Consequently, the traffic levels, once rectified, reflect a standard business hour of a standard business day, instead of the actual time of the observation.

The two rectification profiles obtained are illustrated by graphics in the country summary sheets. The calculation tables with all the successive H-rectification and D-rectification stages can be consulted at any time in the "Distributions" sheet.

In practice, this is of no concern to the calculation workbook user as all the calculations are made directly without any action on his part.

Rectification conditions

This rectification process will only apply to SOURCE measurement campaigns planned and implemented in accordance with the rules set out in this handbook.

Where necessary, it is possible to view the impact of rectification on the indicators, as a simple checkbox at the top of the “RecapSheet” enables you to deactivate the rectification algorithm and to calculate the indicators on the basis of "as is" traffic.

Rectifying prohibited?

If you carry out a SOURCE campaign on a very small country – such as a small island-state in the Indian Ocean – without any special precautions, the reference network will be entirely measured in 3 or 4 days. The ordinary traffic level rectification mechanism, based on campaign observations (and only these observations) will not be valid. It will therefore be necessary either to extend the observation of the traffic levels alone until an adequate standard of coverage is achieved, or to make the combined speed/traffic measurements while complying with very narrow hour/day time slots that have been well chosen at the start.

These are special cases that require the method to be carefully adapted and possibly handled slightly differently. The "as is" workbook, designed for the most common cases, will simply issue a warning.

Rectification essential to the links

The secondary indicator of the network, which is unaffected by traffic, is not modified by rectification. The primary and tertiary indexes are only marginally modified owing to a statistical balancing effect (with a good campaign volume, less than 1% variation). But its impact is highly significant for each link, and this is why rectification is so important.

The traffic distribution map over the network would be worthless without rectifying the traffic levels.

To find out more…

The dual H-rectification and D-rectification algorithm is described in detail in a technical note "Rectifying traffic levels" in the SOURCE CD-ROM. You will find a description of the traffic behaviour model to be used, the conditions of use and a more detailed assessment of the impact of rectification. The same information is provided in full in the "Help" facility  of the calculation workbook..

7. OUTPUTS

The country summary sheets and the speed/traffic maps are calculated automatically. These results, which are the final product of the campaign, aggregate the information originally required  for a standard "fact sheet" of the network.

7.1. Country summary sheet

This sheet, called "SummarySheet" is calculated automatically and compiles the most significant network-wide results in the following compact form:

- a standard set of indexes including the 3 SOURCE macro indicators (for the entire network broken down into paved/unpaved surfaces)

- speed pyramids, illustrating the distribution of travel speeds over the network, with reference to the length, on the one hand, and the traffic volume, on the other hand (for the entire network broken down into paved/unpaved surfaces)

- effective service level ranges, in 4 classes (A, B, C, D) corresponding to speed ranges specific to paved roads on the one hand (more than 80 km/h, 60 to 80 / 30 to 60 / less than 30), and to unpaved roads on the other hand (more than 55 km/h / 40 to 50 / 20 to 40 / less than 20)

- National LV traffic variation profiles, over a day and over a week

7.2. Quality sheet

This sheet enables the relevance of the results to be gauged.

Simple data errors will have been refused on data entry. But more complex errors, that do not prevent calculations, may subsist and affect the statistical relevance of the results. Errors of this type are systematically listed in a special sheet in the workbook, called "QualitySheet". An evaluation of their extent is available (e.g. % of runs involved, % of links involved, % of length involved, with the corresponding link and section codes). In some cases, it is expressly stated whether or not the final indicators are affected to the extent of not being significant.

The following types of errors are listed:

- simple errors in using the workbook

- calibration: transfer errors in the office, data entry errors in the field

- measuring runs: transfer errors in the office, data entry errors in the field

- failure to comply with protocol in the field

- failure to comply with the general area of validity of the method

This gives an immediate quality report, which reflects all the automatic controls performed on the data (but no more: only errors that leave a trace in the workbook data are detected. It by no means dispenses you from the prior control stages).

A very visible warning is added in the summary sheet as soon as the quality report is no longer empty, whatever the number of listed errors.

Note: The workbook provides all facilities for a real assessment of the impact on the indicators of some well-delimited errors. It enables you to replace erroneous values by the minima/maxima to view the discrepancies.

7.3. Speed-traffic maps

It is now easy, after making the calculations in the workbook, to generate a series of standard SOURCE maps that give a summary presentation of the reference network status with respect to LV travel times and LV traffic levels. This part of the processing, which is entirely automatic, is performed by the map editor, which is thus used a second time in the processing chain (because it is assumed that the graphical diagram of the reference network has previously been entered into the map editor, as suggested in chapter 1, parag. 2.4 et seq.). Data transfer from the calculation workbook is also automatic. Apart from prior digitizing of the network, this only takes a few seconds.

The available maps can be edited in a hardcopy version or exported in various computerized formats (drawings or Excel diagrams or various image formats). When displayed through the editor, you can click on them on the screen, link by link1. To display the basic characteristics and corresponding standard values (traffic level and volume and travel speed).

Specimen of map from the Editor

The standard set of maps (the "photograph" of the network) completes the country summary sheet.

1 The map editor does not provide display for sections, as statistical quality is only ensured at link level.