| - PEDAGOGICAL BASICS | - T&T |
| --- | --- |
| ###### The concept of road asset value ###### 1/2 | ###### 5.8 |

- Road network is an economic investment, a collective work tool. The interest of all parties is to prolong its life so as to guarantee debt redemption. If it deteriorates too quickly, it cannot generate the economic benefits intended… but repayment of the initial loan continues. Capital depreciation or asset valuing do not appear in State Budget accounting, but they are strong economic realities. - The asset value is maximum when the road is a new construction. The deterioration speed depends on traffic and climatic events or type of usage (non respect of wet season barriers, and/or unauthorized axle loads) but also, naturally, depends on the quality and frequency of routine and periodic maintenance. - For calculating road asset value at country level, you have to consider all the road sections which are still "maintainable" and serviceable (which should receive periodic or routine maintenance, as opposed to networks needing rehabilitation or reconstruction where the poor state of the roads would make any maintenance action uneconomic). Depending the valorization method, you can possibly take account of non-maintainable roads too, but their value will be very low (or zero).

è

| - PEDAGOGICAL BASICS | - T&T |
| --- | --- |
| ###### The concept of road asset value ###### 2/2 | ###### 5.8 |

è

- Routine maintenance works do not increase asset value (it just lower its drop). Despite the higher costs, nor does periodic maintenance (it is exactly the same). Maintenance permits the asset value to keep its regular declining curve throughout the road design life. By lack of maintenance (compared with the proper technical schedule), the asset value would dramatically fall under this design curve. - Proper maintenance costs each year an average 2% to 5% of the initial road construction cost. For example, for a paved road, if the cost of a new construction is estimated at 100, routine maintenance should cost 2 each year, periodic maintenance should cost 15 every 8 to 12 year, but the cost of rehabilitation should be about 25 as soon as needed due to failing maintenance.