| PEDAGOGICAL BASICS | T&T |
| --- | --- |
| ###### Curvology : « a collection of saws » 1/3 | 5.10 |

|  | Courtesy of UN-ECLAC. See Table of Contents for reference. The deterioration of a road can easily be shown graphically. The road deterioration curve begins at a high point, expressing the very good condition of a new road after its original construction. The curve drops slowly at first, then gains speed until it reaches a bottom point, representing the extremely bad state of a destroyed road. The road agency normally intervenes before the road collapses completely, either by renewing its surface or, if it is too late for that, by rehabilitation or reconstruction. After the intervention the road is again very good. This is shown in the figure by an abrupt rise of the curve to the initial, high level. Over time the entire process is repeated and the curve takes the form of an inverted saw in which the teeth represent the successive interventions improving road conditions.  Road Condition General form of the road cycle Time (years)  |  |  |
| --- | --- | --- | --- |
| PEDAGOGICAL BASICS |  | T&T |  |
| ###### Curvology : « a collection of saws » 2/3 |  | 5.10 |  |

|  | Courtesy of UN-ECLAC. See Table of Contents for reference. The exact form of the saw can vary considerably according to the various factors affecting the road: - The teeth can be more or less distant from each other, showing the speed of deterioration, and depending on the volume of traffic, climate, quality of original construction and of the routine maintenance carried out. - Rapid deterioration cycle Slow deterioration cycle - The space between the teeth can be more or less deep, showing the maintenance policy followed; this can be proper conservation policy (taking measures while the road is still adequate) or rehabilitation / reconstruction policy (taking measures when the road is in a bad or very bad condition). « Conservation policy » cycle « Reconstruction policy » cycle |  |  |
| --- | --- | --- | --- |
| PEDAGOGICAL BASICS |  | T&T |  |
| ###### Curvology : « a collection of saws » 3/3 |  | 5.10 |  |

In the « reconstruction policy » cycle, there are periods when road users have to bear high costs of vehicle operation, because of the bad or very bad state of the road (shaded areas of the curve hereabove). To illustrate what should NOT be done, the curve is shown here for the “ forbidden option ”, where the measures taken are results of emergencies and are usually of poor quality and short-lived. “ Forbidden option ” cycle Apart from the curves shown and described, there are an infinite number of other curves and cycles showing various decisions made in practice. Real is “ a collection of saws ”.

s

Courtesy of UN-ECLAC. See Table of Contents for reference.